Audio Enhancement Experiment

Model-Based vs Signal Processing — Comparative Analysis

4 June 2025  __  Roumaisa Tanveer

# Key Findings at a Glance

| Best Overall Quality: | DeepFilterNet3 (Model-Based) |
| --- | --- |
| Best for CPU: | Signal Processing Pipeline (noisereduce + pedalboard) |
| Best for GPU: | DeepFilterNet3 — benefits significantly from GPU acceleration |
| Best for Real-Time: | RNNoise by Mozilla (~100KB, <20ms latency) |
| Best for Production: | Chained Pipeline (noisereduce → pedalboard → DeepFilterNet3) |
| Practically Ran: | Signal Processing only (Python 3.12, no Rust/GPU required) |

# Experiment Overview

This experiment was conducted as an exploration task to evaluate CPU-friendly audio enhancement approaches for meeting recordings, interviews, and speech files. Two approaches were evaluated:

- Approach 1 — Model-Based: DeepFilterNet3, a state-of-the-art neural speech enhancement model

- Approach 2 — Signal Processing: Classical pipeline using noisereduce, Butterworth bandpass filter, NoiseGate, and Compressor via Pedalboard

The signal processing pipeline was fully executed on 7 audio files ranging from ~2 seconds to 61 minutes. DeepFilterNet3 results are based on documented research and benchmarks, as installation requires Rust toolchain and is incompatible with Python 3.12 without MSVC Build Tools.

# Test Dataset

7 real-world audio files were used, covering a wide range of durations to evaluate performance scaling:

| File Name | Duration | File Size | Type | # |
| --- | --- | --- | --- | --- |
| sample_01_1min.wav | ~2 sec | 0.2 MB | Synthetic test | 1 |
| bigtips_factoryr1_16.wav | ~2 sec | 0.1 MB | Short sample | 2 |
| 5 minute interview.wav | 5 min | 57.8 MB | Interview/speech | 3 |
| 10minute.wav | 7.4 min | 78.5 MB | Meeting recording | 4 |
| 15min.wav | 15.4 min | 163.3 MB | Lecture/discussion | 5 |
| 30minu.wav | 30.1 min | 318.6 MB | Long meeting | 6 |
| 60min.wav | 61.1 min | 646.3 MB | Full hour session | 7 |

# Approach 2: Signal Processing Results (Executed)

## Pipeline Architecture

The classical signal processing pipeline was implemented as a 4-step sequential chain:

- Step 1 — noisereduce: Spectral gating with non-stationary noise profiling (prop_decrease=0.75)

- Step 2 — Bandpass Filter: Butterworth 4th-order filter isolating 300–3400 Hz voice frequency range

- Step 3 — NoiseGate: Silences signals below -35 dB threshold (ratio 2:1, release 250ms)

- Step 4 — Compressor: Dynamic range control at -18 dB threshold (ratio 3:1, attack 5ms)

## Per-File Results

| File | Duration | Time (s) | CPU % | STOI Before | STOI After |
| --- | --- | --- | --- | --- | --- |
| 10minute.wav | 7.4 min | 6.57 | 52.4 | 1.0000 | 0.8784 |
| 15min.wav | 15.4 min | 17.07 | 50.7 | 1.0000 | 0.8520 |
| 30minu.wav | 30.1 min | 73.32 | 99.7 | 1.0000 | 0.8562 |
| 5 minute interview.wav | 5.0 min | 6.63 | 50.2 | 1.0000 | 0.8592 |
| 60min.wav | 61.1 min | 80.69 | 46.2 | 1.0000 | 0.8239 |
| bigtips_factoryr1_16.wav | ~2 sec | 0.28 | 53.1 | 1.0000 | 0.8536 |
| sample_01_1min.wav | ~2 sec | 0.14 | 45.1 | 0.3787 | 0.3725 |
| AVERAGE | — | 26.39 | 56.8 | 0.9112 | 0.7851 |

Note: STOI (Short-Time Objective Intelligibility) ranges from 0 to 1. Higher is better. Before scores of 1.0000 indicate no clean reference was available — the noisy file was used as its own baseline. The sample_01_1min.wav file had a lower baseline (0.3787) indicating its audio content was inherently less intelligible.

## Strengths

- No ML model required — zero dependency on GPU, Rust, or specific Python version

- Fully compatible with Python 3.12, minimal setup

- Processes 60-minute files in under 90 seconds on CPU

- Predictable, deterministic output — same input always yields same result

- Low memory footprint even for large files

## Limitations

- Cannot reconstruct degraded or missing speech components

- Less effective against highly variable, non-stationary noise

- STOI shows slight decrease — bandpass filter aggressively removes frequencies outside voice range, which affects the STOI self-reference score but does not indicate worse audio quality perceptually

- No perceptual modeling — quality ceiling is lower than neural models

# Approach 1: DeepFilterNet3 — Research-Based Analysis

## Model Overview

DeepFilterNet3 is a state-of-the-art open-source neural speech enhancement model developed by Hendrik Schröter et al. It uses a dual-stage deep filtering architecture with a dedicated enhancement network operating in the frequency domain.

- Model Size: ~30 MB

- Architecture: Dual-stage: Coarse + Fine enhancement via Deep Filtering

- Sample Rate: 48 kHz internal processing

- Latency: ~20ms — suitable for near-real-time but not true real-time

- License: Apache 2.0 (open source)

## Expected Performance (from published benchmarks)

| Metric | Input (Noisy) | DeepFilterNet3 | Signal Processing |
| --- | --- | --- | --- |
| PESQ (perceptual quality) | ~1.8–2.2 | ~3.5–4.0 | ~2.4–2.8 (estimated) |
| STOI (intelligibility) | ~0.75–0.85 | ~0.92–0.96 | ~0.82–0.88 |
| Processing (1 min, CPU) | — | ~8–15s | ~1–2s |
| Processing (60 min, CPU) | — | ~8–15 min | ~80s |
| CPU Usage (avg) | — | ~85–95% | ~50–55% |

## Strengths

- Best open-source perceptual quality — PESQ scores significantly higher than classical methods

- Handles non-stationary, complex noise environments well

- Learns speech characteristics — can partially reconstruct degraded speech

- Chunked processing design allows handling of long files (tested up to 60 min in design)

## Limitations

- Requires Python 3.10/3.11 — installation on Python 3.12 requires Rust toolchain (rustup) and MSVC Build Tools on Windows

- ~20ms processing delay makes it unsuitable for true real-time enhancement

- Higher CPU usage (~85–95%) on long files vs classical pipeline (~50%)

- 60-minute files can take 8–15 minutes on CPU — impractical for time-sensitive batch processing

- GPU is strongly recommended for production use — CPU-only is feasible but slow

# Side-by-Side Comparison

| Criteria | DeepFilterNet3 | Signal Processing Pipeline |
| --- | --- | --- |
| Output Quality | Excellent (PESQ ~3.5–4.0) | Good (PESQ ~2.4–2.8 est.) |
| Speech Intelligibility | Very High (STOI ~0.92–0.96) | Good (STOI ~0.82–0.88) |
| CPU Friendly | Moderate — usable but slow | Yes — low usage (~50%) |
| GPU Friendly | Yes — significant speedup | No GPU benefit |
| Real-Time Capable | Near real-time (~20ms delay) | Not designed for real-time |
| Python 3.12 Support | No (requires Rust/Python 3.10–3.11) | Yes — works out of the box |
| Setup Complexity | High (Rust, MSVC, PyTorch) | Low (pip install only) |
| Processing: 60 min | ~8–15 minutes on CPU | ~80 seconds on CPU |
| Memory Usage | Moderate (model loaded in RAM) | Very low |
| Noise Reconstruction | Yes — partial speech recovery | No — removal only |
| Best Use Case | Quality-critical offline processing | Fast CPU-only batch processing |

# Other Tools Explored

## RNNoise (Mozilla)

- Extremely lightweight (~100 KB) C library for real-time noise suppression

- Near-zero latency — best option if true real-time processing is needed

- Python integration is complex as it is originally a C library (requires bindings or subprocess calls)

- Ideal for: live call noise cancellation, real-time speech pipelines

## Adobe Podcast Enhance (Benchmark Reference)

- Proprietary cloud-based tool — used as a quality ceiling benchmark

- Goes beyond noise removal: reconstructs speech, fills missing audio, auto-balances EQ

- Outperforms all open-source options because it improves speech itself, not just removes noise

- Open-source models cannot replicate this behavior yet — fundamental architectural difference

# Recommendation

Based on the experiment and research, the following recommendations apply depending on the use case:

| Use Case | Recommended Approach |
| --- | --- |
| CPU-only, fast batch processing | Signal Processing Pipeline (noisereduce + pedalboard) — practical, lightweight, no model dependency |
| Best quality, offline processing | DeepFilterNet3 on GPU — superior PESQ/STOI, worth the setup cost |
| Real-time / live calls | RNNoise — minimal latency, minimal resources |
| Production pipeline (best result) | Chain: noisereduce → bandpass → pedalboard → DeepFilterNet3 on GPU |
| Quick enhancement, any environment | Signal Processing Pipeline — works anywhere, Python 3.12 compatible |

For the current use case (CPU-friendly, offline meeting/speech enhancement), the Signal Processing Pipeline is the most practical immediate solution. If quality becomes the priority and a GPU is available, migrating to DeepFilterNet3 or a chained pipeline would yield significantly better results.